5-Apr-86 14:51:29-MST,4792;000000000001

Return-Path: <@USC-ISIF.ARPA:info-ada-request@usc-isif.arpa>

Received: from USC-ISIF.ARPA by SIMTEL20.ARPA with TCP; Sat 5 Apr 86 14:51:20-MST

Received: FROM berkeley BY USC-ISIF.ARPA WITH TCP ; 5 Apr 86 13:10:48 PST

Received: by ucbvax.berkeley.edu (5.48/1.11)

id AA01699; Sat, 5 Apr 86 12:34:03 PST

Date: 4 Apr 86 13:36:35 GMT

From: sftig!sfsup!sfmag!sftor!cw@ucbvax.berkeley.edu  (C.S.Wetherell)

Organization: AT&T Bell Laboratories, Summit N.J.

Subject: Two characteristic programming errors in Ada

Message-Id: <951@sftor.UUCP>

Sender: postmaster@usc-isif.arpa (fix your mailer)

Real-Annoyed-Sender: usenet@ucbvax.berkeley.edu

Errors-To: <info-ada-request@usc-isif.arpa>

To: info-ada@usc-isif.arpa

While porting some programs from the Simtel20 archives to our new

compiler, I had some difficulty with one of them.  Upon investigation,

I discovered two subtle but characteristic misuses of Ada.  I am sure they

are quite common, so I will protect the author.

Consider the following procedure declaration (ignore its innards for

the moment)

procedure Does_A_Check(Formal : in out NATURAL);

and the skeletal calling routine

with Does_A_Check;

procedure Im_A_Caller is

Actual : NATURAL;

begin

Does_A_Check(Actual);

end;

When this pair was linked and executed, a constraint exception was raised at

the point of call.  [This happened to be particularly mystifying in the

ported program, because the program was carefully taking care of all

exceptions and I couldn't tell at first exactly what was causing the

program to fail. Some embedding of trace statements uncovered the

difficulty after a bit.]  Of course, the reason the constraint error

was raised was that variable Actual in Im_A_Caller was not

initialized; there is no requirement that the initial value of

a variable be within its declared bounds (with exception of access

types, of course).  Since formal in-out parameters are checked

for constraints at procedure entry, a constraint error was raised.

I can only assume that the particular program had always been compiled

and executed on a system where the initial values were within bounds;

this is, of course, an example of a bug common to other languages as

well.

However, there is a second bug in the program.  Let us now

consider the body of Does_A_Check:

procedure Does_A_Check(Formal : in out NATURAL) is

begin

Formal := 0;

...

Something := ... Formal ...;

...

end;

Notice that the first thing Does_A_Check executes is an initializing

assignment to Formal; the routine really doesn't care about the

in-bound value of the actual argument at all.  In fact, the only

reason that Formal is declared "in out" is so that it can be used

on the right-hand side of some statements.  Now, the Ada rule is that

an "out" parameter cannot be so used; the parameter may have an

undefined value at some places of such usage and the rules about

no right-hand usage keep the compiler from having to do a flow

analysis to determine if a particular usage is after a definition.

Here, the programmer, to save declaring a local variable, just made

Formal "in out"; then it can be used on the right.  But the cost is

unexpected constraint errors or extra work for the caller;

after all, the caller doesn't know (in theory) why the formal parameter

is "in out" and may have to go to considerable trouble to guarantee that

the actual argument has an acceptable value; this is disturbing when

the formal doesn't really need the initial value.

The moral seems to be to use "in out" parameters only when the

called routine actually needs the initial value; otherwise, use

a paradigm like

procedure Does_A_Check(Formal : out NATURAL) is

Local : NATURAL;

begin

Local := 0; -- yes, this could be done in the declaration.

...

Something := ... Local ...

...

Formal := Local;

return;

end Does_A_Check;

Finally, I might note that this problem was uncovered partly because our

compiler temporarily is storing characters in 16-bits while a little

part of our code generator is undergoing additional education about

the differences between signed and unsigned bytes.  One version of the

problem was using characters instead of NATURAL's. If the Actual

from above had been an 8-bit character, it might have been automatically

truncated to within the constraints; as a 16-bit character, it was not.

There is no guarantee in Ada about the sizes of types beyond the rules

governing type declarations; this particular stretch of code was

mildly unrobust in the face of changing sizes

Charles Wetherell

AT&T Information Systems

Summit NJ

#############################