DSP STARTERS KIT

"DSK"

INSTALLATION GUIDE

CONTENTS

--------

- KIT CONTENTS

- INCLUDED ON THE DISKETTE

- CONNECTING THE DSK TO YOUR PC

- RS232 CABLES

- POWER SUPPLY

- SETTING UP THE DSK SOFTWARE

- SPECIFICATIONS

- SCHEMATIC

- BOARD LAYOUT

KIT CONTENTS: Your kit should contain the following materials.

--------------------------------------------------------------

- DSK Installation Guide

- C2x Users Guide

- Applications Diskette (contents)

File           Revision    Description

----           --------    -----------

DSKA     EXE   1.00        DSK Assembler

DSKD     EXE   1.00        DSK Debugger/Monitor

DSKSPEC  DSK    --         DSK Spectrum analyzer (DSK executable)

DSKSPEC  ASM    --         DSK Spectrum analyzer assembly source

DSK_TWID ASM    --         Twiddle tables for DSK Spectrum Analyzer

MMREGS   ASM    --         ASM Include file defining constants

C26_BFNL ASM    --         TMS320C26BFNL bootloader source code

C26_BFNL DOC    --         TMS320C26BFNL bootloader document

INSTALL         --         This file

README   TXT    --         Users guide and additional information

All software is Copyright 1992 Texas Instruments, Inc.

Texas Instruments authorizes that this software can be freely copied

and distributed IF:

- No Fee is charged for use, copying or distributing

- It is not modified in any way

The DSK software is updated and distributed free of charge from

the TMS320 DSP BBS: (713)-274-2323, No parity, 8 data bits, 1 stop bit,

1200/2400/9600 baud.

- DSK Application circuit board

- TMS320C26 DSP processor

- TLC32040 Analog Interface Circuit (AIC)

- RS-232 interface chips

- AC/DC Power supply circuitry

+-------------------------------------------------------------------+

| NOTE: THE RS232 CABLE AND WALL MOUNT TRANSFORMER ARE              |

|         INSTALLATION SPECIFIC AND ARE NOT INCLUDED.               |

+-------------------------------------------------------------------+

| THESE ITEMS CAN BE PURCHASED FROM MOST ELECTRONICS SUPPLY STORES. |

| THE RADIO SHACK CATALOG NUMBERS ARE GIVEN BELOW.                  |

+-------------------------------------------------------------------+

CAT NO.    Description

26-117     9 pin to 9 pin RS232 link

26-240     25 pin to 25 pin RS232 link

26-1388    25 pin to 9 pin RS232 adapter

273-1611A  9 VAC, 1.3 A, UL Class 2 Transformer (uses 2.1mm Jack)

274-1568   2.5mm power jack (for early DSK revisions)

CONNECTING THE DSK TO YOUR PC

-----------------------------

1) Turn off the power to your PC

2) Connect an RS232 extender cable to either COM1 or COM2

3) If needed, connect an RS232 25 to 9 pin adapter

4) Connect the DSK to the RS232 cable

5) Connect the 9 VAC transformer to the DSK

6) Plug the 9 VAC transformer into the wall

7) Turn on the power to your PC

8) Install the DSK software

RS232 CABLES

------------

The DSK requires a 9 pin RS232C connection to the PC host.  If your PC has

a 9 pin connector, all you need is a 9 pin extender cable.  If your comm

port has a 25 pins you will need to have either a cable (same at both ends)

and a 25 to 9 pin adapter or a cable with a 25 pin connector on one end and

a 9 pin connector on the other.

POWER SUPPLY

------------

You will also need a 9 VAC (>250 mA) UL class 2 transformer.  The DSK power

jack accepts 2.1mm (2.5mm in older revisions) plugs which are common to many

wall mount AC transformers.  A low current UL listed transformer is

recommended for the DSK module since they are designed to withstand most

power faults for a brief period of time.

IF YOU HAVE PROBLEMS, IMMEDIATELY UNPLUG THE TRANSFORMER FROM THE WALL

and check that you have everything set up right. In the United States a

suitable transformer (9 VAC, 1.3A) can be purchased from Radio Shack.

(Cat. No. 273-1611A).

SETTING UP THE DSK SOFTWARE

---------------------------

BACK UP THE ORIGINAL DSK DISKETTE AND KEEP IT IN A SAFE PLACE!

Copy the contents of the DSK diskette to a directory of your choice.

If you plan to develop code in another directory you may also want to

add the DSK directory to the path setup statement in AUTOEXEC.BAT

You may want to consult your DOS manual for further details but here

is how it is generaly done.

Edit the PATH statement in AUTOEXEC.BAT to like this...

ECHO OFF

PROMPT $P$G

PATH C:\DOS;C:\MOREDIRS;C:\DSK  <== APPEND ";C:\DSK" TO THE PATH SETUP

Then to make and copy the DSK files to the DSK directory

C:\>MD C:\DSK        <== make a new directory

C:\>CD C:\DSK        <== Change to the new directory

C:\DSK>COPY A:*.*    <== Copy the files to the directory C:\DSK

C:\DSK>DSKD          <== Execute the DSK Debug/Monitor program

At this time the DSK Debugger program (DSKD) will attempt to automaticly

determine the ROM code version and maximum usable baud rate.  If the DSKD is

succesfull a screen will appear with display boxes for the CPU registers,

memory, dissasembly and watch.  If DSKD could not establish communications,

it may display an error message or continually attempt a connection.

Once DSKD has been started try loading the test file DSKSPEC.DSK.  This is

done by hitting the "L" key for load, followed by "D" for DSK in the sub-menu.

Note how the top line changed to the sub-menu when you hit the "L" key.

Next type "DSKSPEC.DSK" into the file to load field, then hit enter.  If

everything worked the file should now be loaded and ready to run.

With DSKSPEC.DSK now loaded into the DSK module, try single stepping the

monitor by hitting the "space" bar.  As you step through the code you should

see the registers and memory being modified by the CPU.  To single step a

set number of times (without hitting "space" all the time) enter "X" for

eXecute, followed by "N" to enter the number of steps. You then enter a value

and hit enter.  To free RUN the DSK, enter "X" for execute, followed by "G" for

Go.  To halt the DSK hit the "ESC" key.

In the event of an error, the COM port, baud rate and ROM revision can

be directly specified on the command line when DSKD is invoked.  The

DSKD invocation help panel can be displayed from DOS by entering

C:\DSK>DSKD ?      <= HINT: Many DOS appliactions use ? for online help

SPECIFICATIONS

--------------

Power Supply:INPUT: 9 VAC, 250 mA minimum, UL Class 2 transformer

OUTPUT: +/-5  VDC regulated, <50 mA

+/-12 VDC unregulated, <50 mA, <4V ripple (no load)

Analog In: +/-3V max

Analog Rin: 100 Kohm typ

Analog Out: +/-3V typ into 600 ohm load

RS232: Requires TX,TR,DTR and GROUND.

RS232 output is +/-5V minimum into 600 ohms

NOTE: The DC supplies are affected by external loading.  Loads in excess of

50 mA may cause excesive ripple that may cause the 5VDC regulators

or RS232 driver to not operate properly.  If you plan to expand the

DSK with external memory, I/O devices etc... you will likely need

to use an external supply.