D TCPIP SLIPNET.DOC

7.  SLIPNET Command Reference

7.1.  Startup

7.2.  Console Mode

7.3.  Commands

7.3.1.  <cr>

7.3.3.  #

7.3.7.  close [<session #>]

7.3.9.  dir [<dirname>]

7.3.11.  echo [accept|refuse]

7.3.12.  eol [unix|standard]

7.3.15.  exit

7.3.17.  ftp <hostid>

7.3.17.1.  abort

7.3.17.2. dir [<file>|<directory> [<localfile>]]

7.3.17.3. get <remote_file> [<local_file>]

7.3.17.4. ls [<file>|<directory> [<localfile>]]

7.3.17.5. mkdir <remote_directory>

7.3.17.6. put <local_file> [<remote_file>]

7.3.17.7. rmdir <remote_directory>

7.3.17.8. type [a|i|l<bytesize>]

7.3.19.  hostname [<name>]

7.3.21.  ip

7.3.21.1.  ip address [<hostid>]

7.3.21.2. ip status

7.3.21.3. ip ttl [<val>]

7.3.22.  memstat

7.3.26.  pwd [<dirname>]

7.3.27.  record [<filename>|off]

7.3.28.  reset [<session>]

7.3.30.  session [<session #>]

7.3.33.  start

7.3.34.  stop

7.3.35.  tcp

7.3.35.1. tcp irtt [<val>]

7.3.35.2.  tcp kick <tcp_addr>

7.3.35.3. tcp mss [<size>]

7.3.35.4.  tcp reset <tcb_addr>

7.3.35.5. tcp rtt <tcp_addr> <rtval>

7.3.35.6. tcp status [<tcb_addr>]

7.3.35.7. tcp window [<val>]

7.3.36.  telnet <hostid>

7.3.39.  upload [<filename>]

7.3.40.  ?

7.4      changes from Phil Karns NET 890421.1 code.

7.  SLIPNET Command Reference

7.1.  Startup

When SLIPNET.EXE is executed without arguments, it  attempts  to  open   the

file  "\GTESLIP.NET"  in  the root directory of the current drive. If it

exists, it is read and executed as though its contents were typed on the

console  as   commands.  This feature is useful for setting the local IP

address and host name, initializing the IP parameters, and  starting

the  various  Internet  services.  If   SLIPNET.EXE is  invoked  with  an

argument, it is taken to be the name of an alternate startup file; it is

read instead of \GTESLIP.NET.

7.2.  Console Mode

The console may be in one of three modes: command mode, converse mode,

and modem mode.

In command  mode, the prompt "slip net>" is displayed and any of the

commands described in the next section may be entered. In converse mode,

keyboard input is processed  according  to the "current session", which

may be either a Telnet or an FTP connection. In a telnet

session,  keyboard input is sent  to the  remote  system  and any output

from the remote system is displayed on the console. In an FTP  session,

keyboard  input is first examined to see if it  is a  known  local  com-

mand; if so it is executed locally. If not, it is "passed  through"  to

the remote FTP server. (See  the  section titled "FTP  Subcommands").

The keyboard also has "cooked" and "raw" states. In cooked  state, input

is  line-at-a-time;  the user may use the line editing characters ^U, ^R

and backspace to erase the line, redisplay the  line   and   erase   the

last  character, respectively. Hitting either return or line feed passes

the complete line up to the application. In raw mode, each character  is

immediately  passed  to  the application as it is typed. The keyboard is

always in cooked state in command mode. It is also  cooked  in  converse

mode  on  an  FTP  session.  In a Telnet session it depends on

whether the remote end has issued (and the local end has  accepted)  the

Telnet "WILL ECHO" option. (See the "echo" command).

On the IBM-PC, the user may escape back to command mode by hitting

the f10 key.

The third mode is modem mode.  You enable this modem from the command

mode by typing the command "modem".  From then on, anything typed will

be send to the commport, and anything received will be diplayed.  There

is no local echo, thus the modem or the host you are connected must

supply it.  To get out of modem mode, hit f10.  Be advised, when NOT

in modem mode, anything coming over the commport is assumed to be IP

data and will be interrupted (or attempted to be interrupted) by the

IP routines.  Conversely, when in modem mode, anything received will

be sent to the screen and not analyzed by any IP routines.

The modem mode is mainly used for setting up the modem and initiating

dial-ups to hosts.  Once you are connected to the host, you may then

drop back into command mode and commence "telnet" or "ftp" sessions.

7.3.  Commands

This section describes each of the commands recognized while in  command

mode.   Note   that  certain  FTP subcommands, e.g., put, get, dir, etc,

are recognized only in converse mode with the appropriate FTP  session;

they   are   not  recognized  while in command mode. The notation "<hos-

tid>" denotes a host, which may be specified in  one  of  two

ways:  as a  symbolic name  listed  in the  file "\hosts.net", or as a

numeric IP address in dotted  decimal  notation  enclosed by  brackets,

e.g.,  [44.0.0.1].

7.3.1.  <cr>

Entering a carriage return (empty line) while in command mode  puts  you

in  converse   mode  with the  current  session. If there is no current

session, SLIPNET remains in command mode.

7.3.3.  #

Commands starting with the hash mark (#) are ignored. This   is   mainly

useful for comments in the \GTESLIP.NET file.

7.3.7.  close [<session #>]

This  command sends a FIN (i.e., initiates a close) on

the session's TCP connection.  This is  an  alternative  to  asking  the

remote server  to initiate a close ("QUIT" to FTP, or the logout command

appropriate for the remote system in the case of Telnet).  When  either

FTP or Telnet  sees the  incoming half  of  a  TCP connection close, it

automatically responds by closing the outgoing half of  the  connection.

Close  is more  graceful than  the "reset"  command,  in  that  it  is

less  likely to leave the remote TCP in a "half-open" state.

7.3.9.  dir [<dirname>]

List the contents of the specified directory on  the   console.   If  no

argument is given, the current directory is listed.

7.3.11.  echo [accept|refuse]

Displays or changes the flag controlling client  Telnet's response to  a

remote WILL ECHO offer.

The Telnet presentation protocol specifies that  in  the  absence of  a

negotiated  agreement   to   the contrary,  neither  end  echoes  data

received from the other.  In this mode, a Telnet client  session  echoes

keyboard  input  locally  and  nothing  is  actually sent until a car-

riage return is typed. Local line editing is also performed:  backspace

deletes  the last character  typed,  while  control-U deletes the entire

line.

When communicating from keyboard to keyboard  the standard  local  echo

mode  is used,  so the setting of this parameter has no effect. However,

many timesharing systems (e.g., UNIX) prefer to do their  own  echoing

of  typed input.  (This makes  screen editors work right, among other

things). Such systems send a Telnet WILL ECHO offer  immediately  upon

receiving an  incoming  Telnet  connection request. If "echo accept" is

in effect, a client Telnet session will automatically return a DO ECHO

response. In this mode, local echoing  and  editing  is turned  off  and

each  key stroke is sent immediately (subject to  the  Nagle  tinygram

algorithm in TCP).  While this mode is just fine across an  Ethernet, it

is  clearly  inefficient  and  painful across  slow  paths  like  packet

radio  channels.  Specifying  "echo refuse" causes an incoming WILL ECHO

offer  to be answered with a  DONT  ECHO;  the  client  Telnet  session

remains  in  the  local  echo mode.  Sessions already in the remote echo

mode are unaffected. (Note:  Berkeley  Unix has a bug in that it  will

still  echo input even after the client has refused the WILL ECHO offer.

To get  around  this  problem,  enter  the  "stty -echo" command to  the

shell once you have logged in.)

7.3.12.  eol [unix|standard]

Displays or changes Telnet's end-of-line behavior when in remote  echo

mode.   In   standard   mode,  each  key  is  sent  as-is. In unix mode,

carriage returns are translated to line  feeds.   This  command  is  not

necessary with   all   UNIX   systems; use  it only  when  you  find

that a particular system responds  to  line  feeds  but not  carriage

returns. Only  Sun UNIX release 3.2  seems  to  exhibit this behavior;

later releases are fixed.

7.3.15.  exit

Exit the SLIPNET program and return to MS-DOS.

7.3.17.  ftp <hostid>

Open an FTP control channel to the specified  remote  host   and enter

converse mode  on the  new  session.

When in converse mode with an FTP server, everything typed on the  con-

sole   is first  examined  to  see if it is a locally-known command. If

not, the line is passed intact to the remote server on the control chan-

nel.   If it  is one of the following commands, however, it is executed

locally. (Note that this generally involves other commands being sent to

the  remote  server on the  control  channel.) When  actively  transfer-

ring  a  file,  the only acceptable command is "abort"; all  other  com-

mands  will  result  in  an  error  message.  Responses  from the remote

server are displayed directly on the screen.

7.3.17.1. abort

Aborts a get, put or dir operation in progress. When receiving a file,

abort  simply  resets the data connection; the next incoming data packet

will generate a TCP RST (reset) in response which will clear the  remote

server.   When   sending a file, abort sends a premature end-of-file.

Note that in both cases abort will leave a partial copy of the  file  on

the  destination  machine,   which   must be  removed manually if it is

unwanted. Abort is valid only when a transfer is in progress.

7.3.17.2. dir [<file>|<directory> [<localfile>]]

Without arguments, "dir" requests that a full directory listing  of  the

remote server's current directory be sent to the terminal.  If one argu-

ment is given, this is passed along in the LIST command; this can be  a

specific file or  subdirectory  that  is meaningful to the remote file

system. If two arguments are given, the second is taken  as  the local

file  into  which the  directory listing should be  put  (instead  of

being sent to the console).  The PORT command is used  before  the  LIST

command is sent.

7.3.17.3. get <remote_file> [<local_file>]

Asks the remote server to send the file specified in the first argument.

The  second   argument,  if  given,  will be the name of the file on the

local machine; otherwise it will have the same name  as  on  the  remote

machine.  The  PORT  and RETR commands are sent on the control channel.

7.3.17.4. ls [<file>|<directory> [<localfile>]]

ls is identical to the "dir" command except that the "NLST"  command  is

sent  to the  server  instead  of the  "LIST"  command. This results in

an abbreviated directory listing, i.e., one showing only the file names

themselves  without any other information.

7.3.17.5. mkdir <remote_directory>

Creates a directory on the remote machine.

7.3.17.6. put <local_file> [<remote_file>]

Asks the remote server to accept data, creating the file named  in   the

first argument.  The  second argument, if given, will be the name of the

file on the remote machine; otherwise it will have the same name  as  on

the  local  machine.  The PORT and STOR commands are sent on the control

channel.

7.3.17.7. rmdir <remote_directory>

Deletes a directory on the remote machine.

7.3.17.8. type [a|i|l<bytesize>]

Tells both the local client and remote server the type of file  that  is

to  be transferred.  The default is 'a', which means ASCII (i.e., a text

file).  Type length  lines   of   text   in  ASCII  separated  by cr/lf

sequences;  in  IMAGE mode, files are sent exactly as they appear in the

file system.  ASCII  mode should be  used  whenever  transferring  text

between dissimilar systems (e.g., UNIX and MS-DOS) because of their dif-

ferent end-of-line and/or end-of-file conventions.  When exchanging text

files between machines of the same type, either mode will work but IMAGE

mode may be somewhat faster.  Naturally,  when  exchanging   raw  binary

files  (executables,  compressed archives, etc) IMAGE mode must be used.

Type 'l' (logical byte size) is used when exchanging binary  files  with

remote  servers   having oddball   word sizes (e.g., DECSYSTEM-10s and

20s). Locally it works exactly like IMAGE, except that it notifies  the

remote  system  how  large the  byte size is. <bytesize> is typically 8.

The type command sets the local transfer mode  and  generates  the  TYPE

command on the control channel.

7.3.19.  hostname [<name>]

Displays or sets the local host's name (an ASCII string such  as  "ka9q-

pc",  NOT an  IP address).  Currently this is used only in the greeting

messages from the SMTP (mail) and FTP (file transfer) servers.

7.3.21.  ip

7.3.21.1. ip address [<hostid>]

Displays or sets the local IP address.

7.3.21.2. ip status

Displays Internet Protocol  (IP) statistics,  such  as  total   packet

counts   and error  counters of various types.  Also displays statistics

about the Internet Control Message Protocol (ICMP), including the number

of ICMP messages of each type sent or received.

7.3.21.3. ip ttl [<val>]

Displays or sets the default time-to-live value placed  in  each  outgo-

ing   IP  datagram.  This limits the number of switch hops the datagram

will be allowed to take. The idea is  to  bound  the  lifetime  of   the

packet   should  it  become caught  in a routing loop, so make the value

somewhat larger than the diameter of the network.

7.3.22.  memstat

Displays the internal free memory list in the storage allocator.

7.3.26.  pwd [<dirname>]

An alias for the cd command.

7.3.27.  record [<filename>|off]

Opens <filename> and appends to it all data  received  on the   current

session.  Data sent on the current session is also written into the file

except for Telnet sessions in remote echo mode. The  command  "record

off"   stops   recording  and  closes the file. This command is not sup-

ported for FTP sessions.

7.3.28.  reset [<session>]

If an argument is given, force a local reset  (deletion)  of  the AX.25

(AXCB) or TCP  Control  Block  (TCB) belonging to the specified session.

The argument is first checked for validity.  If no  argument  is  given,

the current session,  if any,  is used. This  command  should be used

with caution since it does not inform the remote end that the connection

no  longer  exists.   (In TCP  a  reset (RST)  message will be automati-

cally generated should the remote TCP send  anything  after  a local

reset  has  been  done.

It is used to get rid of a  lingering half-open  connection

after a remote system has crashed.

7.3.30.  session [<session #>]

Without arguments, displays the list of  current sessions,   including

session number,  remote  TCP address and the address of the TCP

control block. An asterisk (*) is shown next to  the  "current"

session; entering  <cr>  at  this point will put you in converse mode

with that session.  Entering a session number as an argument to the ses-

sion  command  will  put  you  in converse  mode  with  that session. If

the telnet server is enabled, the user  is  notified  of an   incoming

request  and  a  session  number  is  automatically assigned.   The user

may then select the session normally to converse with the remote user as

though the session had been locally initiated.

7.3.33.  start

Starts  the  specified  Internet  server, allowing  remote   connection

requests.

7.3.34.  stop

Stops the specified Internet server,  rejecting   any   further   remote

connect requests. Existing connections are allow to complete normally.

7.3.35.  tcp

7.3.35.1. tcp irtt [<val>]

Display or set the intial round trip time estimate, in  seconds,  to  be

used  for new  TCP  connections until they can measure and adapt to the

actual value.  The default is 5 seconds.  Increasing this when operating

over  slow channels  will avoid the flurry of retransmissions that would

otherwise occur as the smoothed estimate settles  down  at  the  correct

value.  Note  that  this command  should  be given  before  servers  are

started in order for it to have effect on incoming connections.

7.3.35.2. tcp kick <tcp_addr>

If there is data on the send queue of the specified tcb,  this   command

forces an immediate retransmission.

7.3.35.3. tcp mss [<size>]

Display or set the TCP Maximum Segment Size in bytes that will  be  sent

on   all outgoing TCP  connect  request (SYN segments).  This tells the

remote end the size of the largest segment (packet) it may send.  Chang-

ing   MSS  affects   only  future connections; existing connections are

unaffected.

7.3.35.4. tcp reset <tcb_addr>

Deletes the TCP control block at the specified address.

7.3.35.5. tcp rtt <tcp_addr> <rtval>

Replaces the automatically computed round trip time in the specified tcb

with  the  rttval  in milliseconds.  This command is useful to speed up

recovery from a series of lost packets since it provides a manual bypass

around  the  normal backoff retransmission timing mechanisms.

7.3.35.6. tcp status [<tcb_addr>]

Without arguments, displays several TCP-level statistics, plus  a  sum-

mary   of all   existing TCP  connections, including TCB address, send

and receive queue sizes, local and remote sockets, and connection state.

If  <tcb_addr>  is specified, a  more detailed dump of the specified TCB

is generated, including send and  receive sequence  numbers  and timer

information.

7.3.35.7. tcp window [<val>]

Displays or sets the default receive window size in bytes to  be  used

by   TCP  when  creating new connections. Existing connections are unaf-

fected.

7.3.36.  telnet <hostid>

Creates a Telnet session to the specified host and enters converse mode.

7.3.39.  upload [<filename>]

Opens <filename> and sends it on the current session as though it  were

typed on the terminal. Valid only on AX.25 and Telnet sessions.

7.3.40.  ?

Same as the "help" command.

7.4   Changes from Phil Karns NET 890421.1 code.

As you have probably noticed, may normal IP commands are missing.  This

is because of the "slip-only" configuration, those commands are no longer

applicable (example: route is no longer needed since EVERYTHING is routed

out the slip port).

7.4.1   Commands no longer applicable.

! and shell (removed to reduce code size)

arp

attach (still there, but syntax and meaning has changed)

ax25

connect

disconnect

escape (not need for this IBM-PC only version)

etherstat

finger

help  (use "?" instead)

log  (all logging is sent to the screen)

mode

mulport

param

route

smtp

trace (removed to reduce code size and not really needed in slip)

udp

7.4.2   New commands.

7.4.2.1	   baud <speed>

The baud command is used to change the commport speed.  It is available

only from the command mode.  Remember after changing the baud to go into

modem mode and issue the "AT" command to the modem.  Most modems use this

command to figure out the new commport baud.

7.4.2.2    modem

This command places the console into modem mode.  See para 7.2 for details.

7.4.2.3    attach port <comm> <mtu>

Configure and attach the commport interface to the system.

<comm> is the number of the commport,  ei COM1:  would be 1.

<mtu> is the Maximum  Transmission  Unit  size,  in  bytes.    Datagrams

larger  than   this   limit   will  be  fragmented  at the IP layer into

smaller pieces.

Examples:

# Attach the commport normally known as "com1" (the first

# controller)

# Outgoing packets larger than 1024 bytes are fragmented.

attach port 1 1024

NOTE: You must use a commport that has a COMxBIOS-type driver

installed.  The code checks for such a driver, and exits if not found

for that particular port.  Also note, only ONE comm port may be attached.

7.4.2.4  play <filename>

The play command reads a text file <filename> into the commport just as if

you had typed it from the console when in modem mode.  After the file

has been "played" to the commport, the console automatically goes into

modem mode for you to either "play" another file or ending it with control-Z

or "/EX" which ever is appropriate for the particular host you are linked

to.  (note: to "play" another file, you must hit f10 to return to command

mode before you enter the new "play" command.)

7.4.2.5  capture <filename>|off

This command creates a new file, or appends an existing file.  Anything

written to the screen will be sent to the file (although cooked to properly

handle <CR><LF> pairs).  This capture is NOT for binary files, only text

files.  To end the capture, hit f10, and issue the command "capture off".