--- INTERFACING THE IC-2AT FOR PACKET ---

To interface an ICOM-2AT to the Kantronics Packet Communicator. You will

have to use an audio transformer to do it right.  (From Kantronics phone

support) White and Black leads from the Packet Controller go to the

primary, the Brown to Secondary, the other secondary lead goes to the 2AT

Mic plug.  They suggest a 1 to 1 transformer, the

-------> WHITE <----- || ------> PTT/MIC ON 2AT

)||(

)||(

)||(

-------> BLACK <----- || ------>BROWN<----+

|

|

------------------------------------------+

White/Black/Brown are from the Packet Controller.  Sent in by Dave  (KE4TZ)

Alexandria, Va

Another way to interface to HT

There have been several items posted recently  on  interfacing  HT's  which

have  a  single wire for both the mike and PTT.  The IC2, IC02, TR2600, etc

all work the same -- they key up when the mike line has  DC  continuity  to

gnd.  The transformer approach posted by KE4TZ in #8123 will work fine, but

there is an alternative cheaper approach too.  Try the following circuit:

TX Audio <----|(-----+-------------O  (tip)

C      |                  Mike plug

>    +-------   (sleeve)

<     |

TNC              R   >    |

PLUG                 <     |

>    |

|     |

PTT line <-----------+     |

|

GND      <-----------------+

Typically the required C is .1 uf, and the R is about 1k, but may  need  to

be  changed  by  experimentation.   With  the  TNC2,  this  can  be  easily

implemented by taking the "low" end of R57 (560 ohms) off ground and taking

it  instead to the PTT line (Drain end of Q10, the VN10KM power FET).  Sent

in by Tom - W3IWI.

BOOK REVIEW

"Get ***CONNECTED to Packet Radio" by Jim Grubbs, K9EI, QSKY

Publishing, Springfield, IL, 208 pp, $12.95.

"Get ***CONNECTED..." is the first book about Amateur packet

radio to be published since "Packet Radio" by Robert Rouleau,

VE2PY, and Ian Hodgson, VE2BEN, (TAB Books, Blue Ridge Summit,

PA) was released in 1981. In the intervening five years, the

packet radio mode of Amateur Radio has undergone drastic changes

and Jim Grubbs' new book brings ham radio literature up to date.

In doing so, Jim has managed to cover the state of packet radio

(albeit, only briefly in some cases) as it exists in 1986.

The book is a good primer for the packet radio novice. It briefly

relates the history of packet radio to give the reader some

insight into how the mode got to its present state. It also

describes most of the current production TNCs to inform the

reader what hardware is available.

The highlight of the book is its explanations of the most often

used TNC commands. This helps the reader to interpret some of the

jargon contained in some TNC manuals. (Note that this book only

explains TAPR TNC 1 and TNC 2 commands. VADCG commands are not

covered, while the GLB PK1 commands are cross-referenced in an

appendix.) In addition, the book explains how to vary the com-

mands to optimize a TNC for different modes of operation (HF vs

VHF, direct connections vs repeater connections, etc).

The book consists of the following chapters:

1. The Land of Aurora (a brief history of packet radio),

2. It's Not Terminal (terminals and computers emulating

terminals),

3. Selecting a TNC,

4. Who's On First (how to install the hardware, set some

parameters, and make your first connection),

5. Beyond the First Connect (how to set other parameters and

use other TNC commands),

6. HF versus VHF,

7. Organizations and Publications (sources of information),

8. Stuffing the Bits (what is a packet?),

9. Networking,

10. The Xerox 820,

11. Store and Forward (the W0/RLI message forwarding

system),

12. PAM - A Low Calorie MSO (personal automatic-answering

mailbox),

13. PACSAT, SAREX, Meteors and More (packets in space),

14. File Transfers,

15. Accessories (other packet radio hardware),

16. The Software Approach (computers emulating TNCs),

17. Communicators Welcome.

There are also appendices in the book that contain a wealth of

information. Included is a glossary of packet-radio terms, a list

of commonly used packet radio frequencies, a description of the

W0RLI packet-radio BBS command set, and lists of sources for

packet radio equipment and information (publications and

organizations).

One criticism: The book lacks an index. It is very frustrating to

read a nonfiction book and later, try to find something without

an index. As I was writing this review, I needed to refer back to

things I remember reading and I had a hard time finding them. The

book's Table of Contents is too general to be any kind of substi-

tute for a good index. If you read this book, it is good idea to

use a highlighting pen to mark items that you may want to refer

to later.

In conclusion, Jim has written a book that needed to be written.

I recommend it highly to anyone who wants to find out what packet

radio is all about and how to best utilize packet-radio hardware

without being overwhelmed by the technical intricacies of the

mode.

From WA1LOU

--- PLACE GRID SQUARES ---

Bill-WA4MMP

Most all vhf/uhf and packet enthusisats are familiar with Grid Squares as a

method  of  describing  station location.  Grid squares are used as contest

multipliers in ARRL vhf contests and as the basis for the coveted VUCC (Vhf

Uhf  Century  Club) award.  The basic Grid Square is 2 degrees in longitude

by 1 degree in latitude, or about 80 x 60 nautical miles at  our  latitude.

As  you  will recall, we are in FM18, Baltimore is in FM19, Salisbury MD is

in FM28, Richmond FM17, and Front Royal, FM08.  To indicate  your  location

more precisely, the addition of 5th and 6th characters will define the sub-

square, measuring about 4 x 3 miles at our latitude.

The sub-square is determined by your lat and long  to  the  nearest  5

minutes  in  longitude,  and 2.5 minutes ib latitude.  To determine the 5th

character:  If your number of degrees longitude is a odd number then,  0-05

is  L, 05-10 K, 10-15 J, 15-20 I, 20-25 H, 25-30 G, 30-35 F, 35-40 E, 40-45

D, 45-50 C, 50-55 B, and 55-60 is A.  If your number of degrees longitude is

an  even number then, 00-05 is X, 05-10 W, 10-15 V, 15-20 U, 20-25 T, 25-30

S, 30-35 R, 35-40 Q, 40-45 P, 45-50 O, 50-55 N, and 55-60  is  M.  The  6th

character  is  determined by the minutes os latitude as follows: 00-02.5 is

A, 02.5-05 B, 05-07.5 C, 07.5-10 D, 10-12.5 E, 12.5-15 F, 15-17.5 G,  17.5-

20 H, 20-22.5 I, 22.5-25 J, 25-27.5 K, 27.5-30 L, 30-32.5 M, 32.5-35 N, 35-

37.5 O, 37.5-40 P, 40-42.5 Q, 42.5-45 R, 45-47.5 S, 47.5-50 T,  50-52.5  U,

52.5-55  V,  55-57.5  W,  and finally 57.5-60 is X.  For example, WA4MMP is

located at 77.17 degrees West and 38.41 degrees North.  By using the  above

information,  my  sub-square  is  IQ,  which  makes  my complete designator

FM18IQ.  This lets others know my position fairly accurately which  may  be

benificial  in  aiming beam antennas or sending traffic on packet.  Further

details on Grid Squares can be found in Jan 83 QST, page 49 & 50.