DISCUSSION OF LAYERED PROTOCOL MODEL

Jim Stevens (Stevens@A.ISI.EDU)

18 December 1987

INTRODUCTION

Recently CMC has distributed a nice multi-colored wall chart showing

the DoD Internet Architecture and the protocols at each layer.

I only list CMC as reference because their chart caused a flurry of

e-mail messages on TCP-IP.  Note that their chart is just the latest of

many papers and reference-type-material to contain the same type of

problems.  Since many of you may not have seen the CMC chart, it is

included for reference as Figure 1 at the end of this message.

There are 2 problems with this chart:

(1)  Location of EGP, GGP, and HMP and

(2)  Incorrect Physical and Data Link protocols under some networks

(especially radio networks).

LOCATION OF NETWORK MANAGEMENT PROTOCOLS

The location of EGP, GGP, and HMP have caused a flurry of e-mail

messages on TCP-IP.  These 3 protocols are network management

protocols.  There has been a lot of work in the last decade on protocol

architectures which include network management that provide an answer

to the question of the network management protocol location.  Some of

the efforts include ISO (reference ISO 7498 Part 4), CCITT (ISDN, see

recommendation I.320), and AT&T (J.W. Timko, AT&T Technology, Vol 2, No

3).  I intend to locate EGP, GGP, HMP, and ICMP within the ISDN

protocol stack using the OSI management structure categories.

All three of the above named efforts basically came up with the same

architecture.  These architectures just added a new dimension to the

current the 7-layer OSI reference model.  The 7-layer OSI model

contains only 1 dimension, which I'll call the X dimension.  The X

dimension described the different services performed upon User data as

it is sent from one host to another.  The new architectures added a Y

dimension which showed the different functions which must be performed

at each layer.  The three different functions that are performed are:

1.  The (N)-layer performs some service upon user data and send the

data onward.  (This function is identical to that described in

the original X dimension.)

2.  The (N)-layer performs signaling between the different layers.

(For packet switched networks this signaling is typically

provided in the packet headers.)

3.  The (N)-layer performs internal management decisions.

Figure 2 shows the ISDN protocol stack and the 2 dimensions of their

model.  The user service function is the vertical stack of layers

labeled with a U.  The signaling function is the vertical stack of

layers labeled with a C.  The management function is the vertical stack

of layers labeled with an M.

The OSI management framework describes 3 management structure

categories:

1.  Systems Management

System Management Application-Process (SMAP)

System Management Application-Entity  (SMAE)

2.  (N)-layer management

coordinates multiple instances of communications

3.  (N)-layer operation

coordinates single instance of communications

The Systems Management is an application process which can use the

entire protocol stack when communicating.  The difference between a

Systems Management application process and another user application

process (such as Terminal Emulation) is that the Systems Management

application has access to the control/management data within each of

the protocol layers.   EGP, GGP, and HMP are system management

applications.

The (N)-layer management/operation are processes within the (N)-layer.

Thus these processes can only use the protocols which lie under them.

ICMP is a Network-layer management process.

INCORRECT PHYSICAL AND DATA LINK PROTOCOLS UNDER SOME NETWORKS

The CMC chart has confused the Physical and Data Link protocols used

within some networks and the Physical and Data Link protocols used to

interface to these networks.  This has been a common problem and has

shown up in several different papers.  Figure 3 shows a layered

protocol chart which did not have this problem.

Since I have worked on the DARPA Packet Radio Program and its follow on

Survivable Adaptive Networks (SURAN) Program, I will use older fielded

Improved Packet Radio (IPR generation Packet Radios) as an example to

explain how the confusion arises.

Normally, the 7 protocol layers are implemented in a single box.  The

protocol drivers are then contained within either the OS kernel or

applications residing above the OS.  This is not true however in Packet

Radio, where the 7 protocol layers are implemented in 3 different boxes

interconnected via a BBN 1822 line.  Figure 4 shows how the DOD

protocols stack is implemented in 3 boxes interconnected together with

BBN 1822.

The Hosts interface to the Packet Radio Network via a Host Interface

Unit (HIU).  The actual interface between the HIU and the Hosts is a

defined host access protocol running on top of BBN 1822.  The hosts

implements the Session and higher layer protocols such as FTP and mail.

The HIU interface to the actual Packet Radios (PRs) using a defined

device access protocol running on top of BBN 1822.  The HIUs implement

the Internet and Transport protocols such as IP and TCP.

The PRs communicate to each other using the PR Channel Access Protocol

(CAP) at the network and upper data link layers, CSMA at the lower data

link layer, and spread spectrum at the physical layer.  The PRs thus

implement the Network, Link, and Physical layers.

There is often confusion between the host or device interface to Packet

Radio and the actual protocols used within Packet Radio.  That is why

the CMC chart shows BBN 1822 as the Packet Radio Network Physical

protocol instead of spread spectrum.

It is fairly easy to catch this type of error for radio networks, since

the radio network physical layer must be some sort of RF transmission

scheme such as Spread Spectrum (SS) or Amplitude Modulation (AM).

------------------------------------------------------------------------------

Figure 1

Layered Protocol Model

Taken from CMC chart, Fall 1987

DOD

MODEL

+-------+ +------+ +-------+  +--------------------------+   +-------+

Appli-  | File  | | Mail | | Term- |  |      User Programs       |   | Name  |

cation  | Trans-| | Text | | inal  |  |                          |   | Server|

|  fer  | | RFC  | | Emul- |  |                          |   |       |

| Server| | 822  | | ation |  |                          |   |       |

+-------+ +------+ +-------+  +--------------------------+   +-------+

^         ^        ^             ^          ^       ^          ^

.         .        .             .          .       .          .

.         .        .             .          .       .          .

v         v        v             v          v       v          v

+-------+ +------+ +-------+ +-----------+ +------+ +-----+  +-------+

Util-   |  FTP  | | SMTP | |Telnet | |   NETBIOS | | TFTP | | NFS |  |  NSP  |

ity     |  MIL  | | MIL- | | MIL-  | |     RFC   | |      | |     |  |       |

|  STD  | | STD  | | STD   | |    1001/  | |      | |     |  |  RFC  |

|  1780 | | 1781 | | 1782  | |    1002   | |      | |     |  |  882  |

+-------+ +------+ +-------+ +-----------+ +------+ +-----+  +-------+

^         ^        ^         ^     ^       ^       ^          ^

.         .        .         .     .       .       .          .

..............................     ............................

.                     .

v                     v

+-----+ +-----+ +-----+   +------------------+    +-------+

Trans-   | EGP | | GGP | | HMP |   |       TCP        |    |  UDP  |

port     |     | |     | |     |   |   MIL-STD 1778   |    |  RFC  |

|     | |     | |     |   |                  |    |  768  |

+-----+ +-----+ +-----+   +------------------+    +-------+

^       ^       ^              ^                   ^

.       .       .              .                   .

.       .       .              .                   .

....................................................

.

v

Inter-         +------------------------------------+----------------+

net-           |       Internet Protocol (IP)       |      ICMP      |

work           |            MIL-STD 1777            |    RFC 792     |

+------------------------------------+----------------+

^

.

.

..............................................

.        .         .             .           .

.        .         .             .           .

v        v         v             v           v

+-----+ +-------+ +---------+ +------------+ +--------------+

Net- | Pkt | |  BBN  | |   Pkt   | | CCITT X.25 | | Ethernet ARP |

Work |Radio| | 1822  | |Satellite| |Packet Layer| |   RFC 826    |

+-----+ +-------+ +---------+ +------------+ +--------------+

^     ^    ^       ^             ^           ^

.     .    .       .             .           .

.     .    .       .             .           .

.......    .........       .......           .

.          .       .       .     .           .

.          .       .       .     .           .

.          v       v       v     v           v

.     +------+ +-----+ +-----+ +-----+ +--------------+ +-------+

Data    .     | BBN  | | BBN | |LAPB | |LAPB | |IP / Ethernet | | IEEE  |

Link    .     | HDH  | | VDH | | BSC | |HDLC | |   RFC 894    | | 802.2

.     |      | |     | |     | |     | +--------------+ +-------+

.     |      | |     | |     | |     |   ^                  ^

.     |      | |     | |     | |     |   .                  .

.     |      | |     | |     | |     |   .   .......................

.     |      | |     | |     | |     |   .   .     .        .      .

.     |      | |     | |     | |     |   v   v     v        v      v

.     |      | |     | |     | |     | +------+ +-----+ +-----+ +----+

.     +------+ +-----+ +-----+ +-----+ |      | |     | |     | |    |

.        ^       ^       ^        ^    |      | |     | |     | |    |

.        .       .       .        .    |      | |     | |     | |    |

.        ..........................    |      | |     | |     | |    |

.        .       .       .        .    |      | |     | |     | |    |

v        v       v       v        v    |      | |     | |     | |    |

+----+ +-----+ +-----+ +------+ +------+ |      | |     | |     | |    |

Phys- |BBN | |CCITT| | EIA | | EIA  | |MilStd| | IEEE | |IEEE | |IEEE | |FDDI|

ical  |1822| |V.35 | |RS449| |RS232C| | 188C | |802.3 | |802.4| |802.5| |    |

+----+ +-----+ +-----+ +------+ +------+ +------+ +-----+ +-----+ +----+

------------------------------------------------------------------------------

FIGURE 2.

ISDN Protocol Stack

Taken from CCITT I.320

Control          User Data        System Management

________         ________         ________

| \       \      | \       \      | \       \

|    \       \   |    \       \   |    \       \

\     \ ______\  \     \ ______\  \     \ ______\

\   |      |     \   |      |   o \   |      |

\|______|        \|______|   o    \|______|

@            @        o    o    @

@            @        o    o    @

@      oooooo@ooooooooo    o    @

@      o     @             o    @

@      o     @   ooooooooooo    @

______@____ _o_____@_  o              @

| \    @     \o     @  \o      @@@@@@@@@

|    \    C   o \   @   o \    @

|       \  ________\@   o    \ @

| \       |\        @ \ o  M   @\

|    \    |   \  U      o\     @   \

|       \ |      \ _________\_________\

| \       |\      |          |         |

|    \    |   \   | 7 - Appl |         |

|       \ |      \|__________|         |

| \       |\      |          |         |

|    \    |   \   | 6 - Pres |         |

|       \ |      \|__________|         |

| \       |\      |          |         |

|    \    |   \   | 5 - Sess |         |

|       \ |      \|__________|         |

| \       |\      |          |         |

|    \    |   \   | 4 - Tran |         |

|       \ |      \|__________|         |

| \       |\      |          |         |

|    \    |   \   | 3 - Netw |         |

______________|       \ |      \|__________|         |

.                \       |\      |          |         |

.                \    |   \   | 2 - Link |         |

.     .                \ |      \|__________|         |

.     .                \      |          |         |

.     .                \   | 1 - Phys |         |

.     . _______________\|__________|_________|

.           Physical Media      |

. ___________________________|

------------------------------------------------------------------------------

Figure 3.

Layered DOD Reference Model Chart

Taken from "Tactical Information Exchange (TIE) Framework

Development (For the 1990s Time Frame" October 1981

+-----+ +----------+                   +------------+

Application            | MTP | |Multimedia|                   | Conference |

|     | |Message   |                   |            |

+-----+ +----------+                   +------------+

^   ^                                  ^     ^

.   .                                  .     .

.   .                                  .     .

.   .                                  .     .

+------+ +---+ .   .    +----+ +------+               .     .

Utility      |TELNET| |FTP| .   .    |TFTP| |Name  |               .     .

|      | |   | .   .    |    | |Server|               .     .

+------+ +---+ .   .    +----+ +------+               .     .

^       ^   .   .       ^     ^                    .     .

.       .   .   .       .     .                    .     .

.       .   .   .       .     .                    .     .

.................       .......                    .     .

.                  .                       .     .

.                  .                       .     .

v                  v                       v     .

+------------------+     +---------+ +-----+  +--------+   .

Transport     |       TCP        |     |   UDP   | | GGP |  | NVP II |   .

+------------------+     +---------+ +-----+  +--------+   .

^                    ^         ^          ^       .

.                    .         .          .       .

.                    .         .          .       .

................................          .........

.                       .

.                       .

v                       v

+-----------------------------------------------+ - - - - - - - - -

Internet  |               Internet Protocol               | Stream Protocols !

+-----------------------------------------------+ - - - - - - - - -

^

.

.

......................................................

.                .          .       .        .       .

.                .          .       .        .       .

v                v          v       v        v       v

+----------+       +------+   +------+ +-----+ +-----+ +------+

Network        | ARPANET  |       |SATNET|   |CCITT | |PRNET| |JTIDS| | DIN  |

| HOST/IMP |       |      |   | X.25 | |     | |     | |2 SIP |

+----------+       +------+   +------+ +-----+ +-----+ +------+

^                ^          ^       ^        ^       ^

.                .          .       .        .       .

.                .          .       .        .       .

..............      .........      .       .        .       .

.      .     .      .       .      .       .        .       .

.      .     .      .       .      .       .        .       .

v      v     .      v       v      v       v        v       v

+----+ +----+   .    +-----+ +----+ +-----+ +-----+ +-----+ +------+

Link      |HDH | |VDH |   .    | VDH | |HDH | |HDLC | |CSMA | |TDMA | |ADCCP |

|HDLC| |RTP |   .    | RTP | |HDLC| |     | |     | |     | |      |

+----+ +----+   .    +-----+ +----+ +-----+ +-----+ +-----+ +------+

^      ^     .      ^       ^      ^       ^        ^       ^

.      .     .      .       .      .       .        .       .

.      .     .      .       .      .       .        .       .

v      v     v      v       v      v       v        v       v

+---+ +-----+ +----+ +-----+ +----+ +-----+ +-----+ +-----+ +------+

Physical  |M/W| |Modem| |BBN | |Modem| |M/W | |X.21 | | SS  | |SS/FH| | M/W  |

|   | |     | |1822| |     | |    | |     | |     | |     | |      |

+---+ +-----+ +----+ +-----+ +----+ +-----+ +-----+ +-----+ +------+

Key:

ADCCP      Advanced Data Communications Control Procedures

DIN 2 SIP  AUTODIN II Segment Interface Protocol

FH         Frequency Hop

HDH        HDLC Distant host

JTIDS      Joint Tactical Information Distribution System

MTP        Mail-Transport Protocol

M/W        Modem / Wire

NVP II     Network Voice Protocol

PRNET      Packet Radio Network

RTP        Real Time Protocol

SATNET     (Experimental) Satellite Network

SS         Spread Spectrum

TDMA       (Synchronous) Time Division Multiple Access

VDH        Very Distant Host

----------------------------------------------------------------------------

Figure 4

Host Interface to Packet Radio

+---------------+

|               |  Host

Application       |FTP, Mail, etc.|

|               |

Utility           |               |

+---------------+

^

.  BBN 1822 and defined host access protocol

.

v

+---------------+

|               |  Host

Transport         |      TCP      |  Interface

|               |  Unit

|               |

Internetwork      |      IP       |

|               |

+---------------+

^

.  BBN 1822 and defined device access protocol

.

v

+---------------+

|               |  Packet

Network           |    Channel    |  Radio

|    Access     |

|   Protocol    |

|               |

|               |

Link              |     CSMA      |

|               |

|               |

Physical          |    Spread     |

|   Spectrum    |

|               |

+---------------+

----------------------------------------------------------------------------

-------