Date: Sun, 5 Aug 90 20:32 GMT+1

From: Andy Bakkers <ELBSCBKS%UTWENTE.NL@CUNYVM.CUNY.EDU>

Subject: Transputers in packet-radio

To: Packet-Radio@UCSD.Edu

THE USE OF TRANSPUTERS IN THE NEXT GENERATION PACKET RADIO HARDWARE

Andre P. Bakkers PA0APA,

PI4THT E.T.G.D. Club Station,

Dept. of Electrical Engineering,

University of Twente,

P.O. Box 217,

7500 AE Enschede, Netherlands,

e-mail: elbscbks @ utwente.nl

1       INTRODUCTION

Traditional microprocessors have to be programmed from scratch. Every

processor requires some kind of an operating system to let you execute a

program. Multi tasking will require again a different operating system and

non of this stuff is standard. Oh I am sorry you want to call Unix a

standard. Go ahead! Now there is this new chip the transputer, it handles

processes instead of assembler instructions. It has a built-in "on chip" round

robin scheduler. Typical time slices are 1 milliseconds and a context

switch (process switch) takes 1 micro second. Oh I forgot HAMS always want

to know the price. A PC plug-in board is $236 without memory. Interested?

Read on!

2       THE TRANSPUTER AND OCCAM

One way out of the traditional computing requirements has has long been

sought in the use of parallel computing. In the past, many attempts to

realize parallel systems with traditional processors have failed due to the

software overhead. This is why the interfaces all require special

hardware or busses, e.g. ethernet, SCSI you name it.

Here is a new concept of communication between processors and processes.

Communication between processors and processes is done by means of

channels. Due to the synchronization properties of these channels, the

restriction to one processor no longer exists. The transputer hardware and

companion Occam software development has implemented this concept. By using

a formal specification language, the correct behavior of the implementation

of the Occam constructs in hardware was formally proven. The

hardware-software integration resulted in the transputer-Occam combination.

The transputer may be considered as a building block for parallel

computers. Occam enables programming across these parallel transputers. In

the following paragraphs the operation of the transputer will be explained,

based on the following list of major characteristics. From this list a

number of significant elements of the transputer chip can be identified.

The following elements have been integrated as one micro computer chip.

1. Communication to other transputers is performed over four

self-synchronizing, high speed (20 Mbit/sec), DMA driven, bi-directional,

serial links.

2. A round-robin process (task) scheduler is implemented in micro-code

realizing a process switching time of 1 microsecond.

3. A floating point processor realizing 1.5 Megaflops.

4. A built-in timer with an accuracy of 1 microseconds in high priority mode

or 64 microseconds in the low priority mode.

5. Fast (50 nano-seconds) internal memory of 4 Kbyte.

6. A 32 bit 10 MIPS CPU.

2.1  The Occam process

The underlying idea of Occam is the notion of a process. A process is a

part of a program that starts, performs an action and then terminates. The

idea is that processes may be executed in parallel. The communication

between processes is done via self-synchronizing channels. A channel is a

one way point-to-point connection from one process to another. The

processes may be located on different transputers. In that case the

communication over channels changes into communication over links. There is

no need for the programmer to worry about the implementation of the channel

or link protocol because it is implemented in micro-code on the transputer

chip.

Examples of so-called primitive processes in Occam are:

x := 3          assigns the value of three to the variable x

channel1 ? p    takes a value from an input channel1 and puts it in p

channel2 ! q    takes the value of q and outputs it over the channel2

An Occam program consists of combinations of primitive processes so that a

number of these primitive processes may be combined into a larger

construction. For this purpose Occam supports the following constructs:

SEQ             To indicate the start of a series of sequential

process1      processes.

process2

PAR             To indicate the start of a series of parallel

process1      processes.

process2

Note that the indentation under the SEQ and PAR constructs are syntactic

and are used to indicate the begin and end of the construct.

2.2  Links and Channels

In Occam communication via links looks as follows:

In transputer A                         In transputer B

we have the code:                       we have the code:

communication

Link.2 ? var.y     <-----------------   Link.2 ! var.b

via Link.2

The input ? and the output ! processes may be considered as the

synchronization between processes running on transputers A and B. If the

process on transputer-A arrives at the input instruction on Link.2 and

transputer-B is not yet ready to provide its output on Link.2, the process

running on transputer-A will be de-scheduled until transputer-B signals

that it is ready to send the data over Link.2. At that point the inputting

process on transputer-A is resumed and the data is transferred via Link.2.

This results in a synchronizing action between the processes running on

different transputers. In this way the transputer links provide the

necessary system interconnection and synchronization. The only thing the

programmer has to do is to define the appropriate input and output actions.

Identical to the link concept, the channel concept is used to communicate

between processes located on the same transputer as illustrated below.

In process A                            In process B

we have the code:                       we have the code:

communication

Chan.1 ? var.x     ----------------->   Chan.1 ? var.a

via Chan.1

This unifying link/channel concept is very convenient during the system

design and check-out phase. Different hardware and software layers can be

developed independently as long as the software and hardware interface

format has been defined.

Inside the transputer the channel/link administration is kept in one memory

word. This memory word will contain the most negative value to indicate

that the channel has not yet been called, or a value that is interpreted as

a workspace pointer of a (de-scheduled) process. The transputer channels

may therefore be considered as a standard system interface because they

provide the necessary system interconnection and synchronization.

2.3  Interrupts

The transputer has an interrupt signal pin connected to the event channel.

This event channel is read identically to a link, although no data can be

transferred. It can only be used to activate a process that is waiting on

an input ? from the event channel. As soon as the event channel is

activated, the waiting process is scheduled by inserting it at the back of

the ready queue in the low priority mode, or it may be scheduled at once as

a high priority process. The use of the transputer interrupt signal is

illustrated in the following Occam interrupt process that is activated

every time the event pin is triggered. The program starts with the

definition of the process name with the external channel definition. The

internal event channel is declared as the type ANY because there is no data

transport over the event channel. This event channel is associated with the

actual hardware pin. This is illustrated in the following program.

PROC interrupt (CHAN OF INT16 output)

CHAN  OF  ANY event:

PLACE event AT event..in:

WHILE TRUE                 <<< Do forever loop!

SEQ                      <<< Start sequential process

event ? any            <<< Wait for interrupt

....execute handler code

:                            <<< Occam terminator of a process

The transputer has one high-priority and one low-priority operating mode.

The interrupt process should preferably be executed in the high priority

mode in order to reduce the interrupt response time.

2.4  The Timer

The timer channel should also be considered an input channel. Times of the

internal running clock are obtained by reading the timer channel and

operating on the time value. The Occam code to use the timer is illustrated

with the following SEQuential process that causes a delay:

PROC delay (VAL INT interval)

TIMER clock:

INT time:

SEQ

clock ? time

clock ? AFTER time PLUS interval

:

Internally the timer queue differs from the ready queue in that it is kept

sorted in the  sequence of the wake up times of the different timed

processes.

2.5  The Scheduler

In addition to the channel protocol concept, there is another important

difference between transputers and other microprocessors. That is the

built-in scheduler. The transputer should be considered as handling tasks

or processes rather than machine instructions. These processes are

controlled by a built-in scheduler. The process administration is the main

activity of the scheduler. The scheduler keeps track of the different

processes by keeping a list of processes in an area of memory that is

called the workspace. The workspace of the current process is pointed to by

the transputer register called workspace pointer. The workspaces are linked

together to form a queue. The scheduler keeps track of four queues, i.e.

for each priority a ready queue and a timer queue. The beginning and end

pointer to a queue are maintained in the transputer registers called Front

pointer and Back pointer. Workspaces are added to the ready queue by

reference to the back pointer and workspaces are extracted by reference to

the front pointer.

2.5.1  High-priority mode

The process (or task) scheduler can operate in one of two modes, i.e. the

high-priority or the low-priority mode. In the high-priority mode the

executing process cannot be interrupted by another process (non-preemptive

scheduling). De-scheduling can only occur when:

o   a process is completed

o   communication with a channel/link is not (yet) possible

o   the process waits for a timer to elapse

2.5.2  Low-priority mode

In the low-priority mode the processes are, by means of time-slicing,

scheduled in a round-robin manner. Here processes will be de-scheduled by:

o   completion of the process

o   awaiting link or channel communication

o   expiration of a time slice

o   a high-priority process becoming ready

The de-scheduled process will be added to the back of the ready queue and

the process from the front of the ready queue will be executed. The time

slice in a transputer is typically 1 msec. and the process switching time

is of the order of 1 microsecond. In Occam the priority may be assigned

using a PRIority addition to the PAR construct.

Without priority:                       With priority:

PAR                                     PRI PAR

process.1                               process.1

process.2                               process.2

The processes 1 and 2 are executed in parallel, either both at the same

priority, or with the PRI PAR:  process 1 will be executed in high priority

and process 2 in low priority. The PAR process is only finished after both

processes have been executed.

3   THE BASIC BUILDING BLOCK METHOD

A systematic way to introduce parallelism is to start with very small

processes and build a system with these elementary processes. This approach

is similar to methods used in past in analog computers. The internal scheduler

on the transputer will realize parallelism while running these processes.

As an example I like to mention the realization of a control program with

the use of only five basic elements. The basic elements in this system

were:

a splitter block:               tee

a gain block:                   gain

a summer block:                 sum

a minus block:                  min

an integrator block:            int

Let's first define these basic elements and then look at the controller

program using these basic elements.

PROC tee (CHAN OF REAL32 in, out0, out1)

REAL32 x :

WHILE TRUE

SEQ                                         /|

in ? x                              in ---/  |----out0

PAR                                       \  |----out1

out0 ! x                                  \|

out1 ! x

:

PROC gain (CHAN OF REAL32 in, out, init)

REAL32 x,k :

SEQ

init ? k                                   |-------|

WHILE TRUE                          in  ---| gain  |--- out

PRI ALT                                    |       |

init ? k                                 |---|---|

SKIP                                       |

in ? x                                       |init

out ! k * x

:

PROC sum (CHAN OF REAL32 in1, in2, out )

REAL32 x1, x2 :

WHILE TRUE                                  |--------|

SEQ                                in1 -----|        |

PAR                                       |  sum   |---- out

in1 ? x1                       in2 -----|        |

in2 ? x2                                |--------|

out ! x1 + x2

:

PROC min (CHAN OF REAL32 in1, in2, in3, out)

REAL32 x1, x2, x3 :

WHILE TRUE                                  |--------|

SEQ                              in1 -----|        |

PAR                                     |        |

in1 ? x1                     in2 -----|  minus |---- out

in2 ? x2                              |        |

in3 ? x3                     in3 -----|        |

out ! -((x1 + x2) + x3 )                |--------|

:

PROC int (CHAN OF REAL32 in, out, init)

REAL32 x,t :   <<< SYSTEM STATE AND TIME

SEQ

init ? x;t <<< INITIALIZE STATE AND TIME

WHILE TRUE                              |-------|

PRI ALT                               |       |

init ? x;t                  in -----| inte- |---- out

SKIP                              | grator|

TRUE & SKIP                         |---|---|

REAL32 dx :                           |

SEQ                                   | init

PAR

in ? dx       -- Input in parallel

out ! x        -- with output

x:= x + (dx*t)

:

These building blocks defined above may now be used to convert for example

a control system defined as a block diagram (think of it as an analog

computer) into an Occam program to run on a transputer. The application

realized in our control laboratory is given below. It is not necessary for

you to study this program it is only given as an illustration. Note

that the insertion of the SPLITTER blocks is necessary because the Occam

channels are point-to-point connections. Installation of a SPLITTER block

looks like a solder joint.

After the definition of the basic building blocks, the complete calculation

process for one control mode looks like the following Occam process:

The channel names have been changed to a vector type representation.

PROC calc ([4]CHAN OF REAL32 in, CHAN OF REAL32 outM,

[13]CHAN OF REAL32 I)

[27]CHAN OF REAL32 D :

PAR

gain ( in[0], D[0],  I[0]  )

gain ( in[1], D[1],  I[1]  )

gain ( in[2], D[2],  I[2]  )

gain ( in[3], D[3],  I[3]  )

min ( D[0],  D[1],  D[2],  D[8] )

min ( D[3],  D[4],  D[5],  D[9] )

sum ( D[6],  D[7],  outM  )

gain ( D[8],  D[12], I[6]  )

gain ( D[10], D[6],  I[4]  )

gain ( D[11], D[7],  I[5]  )

tee ( D[9],  D[13], D[14] )

sum ( D[12], D[13], D[15] )

sum ( D[16], D[14], D[17] )

gain ( D[18], D[5],  I[10] )

gain ( D[19], D[4],  I[11] )

tee ( D[15], D[20], D[21] )

int ( D[17], D[23], I[9]  )

sum ( D[22], D[24], D[26] )

gain ( D[20], D[22], I[7]  )

gain ( D[21], D[16], I[8]  )

tee ( D[23], D[24], D[25] )

tee ( D[25], D[10], D[18] )

int ( D[26], D[27], I[11] )

tee ( D[27], D[19], D[11] )

:

With these instructions the scheduler of the transputer can go to work and

schedule all these parallel processes for execution. The advantage of this

method is the speed of the development of the program. The above program is

for a digital controller.

There are, however, some remarks to be made about this design method.

Because a large number of building blocks are interconnected, there is a

danger of deadlock. It is a property of so-called I/O-parallel blocks that

if combined into networks, they will exhibit deadlock free properties. An

I/O parallel block executes the input and output in parallel and not

sequentially. The careful use of a few I/O-parallel blocks in a network

will result in a deadlock free behavior of the network. For that reason the

integrators used above are I/O parallel as indicated in the corresponding

Occam program, resulting in a deadlock free program.

4       CONCLUSIONS AND SUGGESTIONS

The overall conclusion of the experience gained over a number of projects

is that the application of transputers allows the design of

high-performance systems. For complex systems the basic element method is a

very good approach to realize the parallel programs.

I would like to invite all you TCP IP NET etc. software to suggest which

application according to your opinion would be interesting for packet radio.

Just to make you feel better: it is possible to program a transputer in C,

Pascal or even FORTRAN. It is also possible to cast C program in an Occam

harness so that Occam will take care of the parlellism. But Occam is a nice

parallel language and it illustrates the parallel implementations very well.

If you have any questions please direct them to my email address. I will

try reply in the form of another write-up

73's

PA0APA

------------------------------