EQUIPMENT OPTIONS

FOR MEDIUM TO HIGH-SPEED PACKET RADIO

Compiled by Barry McLarnon, VE3JF

(Last update: 14 Dec 94)

The purpose of the following is to summarize the hardware options

available for constructing medium- to high-speed packet radio links.

The speed range in question is 9600 bps and up. This material is

intended to be a useful reference, but I make no claims as to its

accuracy or completeness. Some details concerning model numbers and

prices are missing, and I have very little information concerning

equipment sources outside North America. If you have corrections, or

suggestions on additional information to include in this survey,

please send them to bm@hydra.carleton.ca (or

ve3jf@va3tcp.#eon.on.can.noam).

This material is released to the public domain. You can do what you

want with it - all I ask is that you retain this notice and

attribution if you reprint it in whole or in part.

Note: unless otherwise noted, prices given are in $US. They should be

considered only as approximate.

EQUIPMENT FOR 9600 BPS

9600 bps Modems

The K9NG modem was available for a number of years as a kit from TAPR.

It set the "standard" for 9600 bps packet operation, but it has now

been replaced by the G3RUH and new TAPR designs. Among the

improvements provided by the newer designs is full-duplex capability.

Even when full-duplex is not needed on the air, this is a great

convenience for doing loop back testing of the modem. If you still

have a K9NG modem lying around, though, don't hesitate to try it.

The G3RUH modem is available from several sources:

PacComm MC-NB96 internal modem card ($115)

fits on disconnect header of most TNCs.

PacComm EM-NB96 external modem ($149)

standalone version of above.

Kantronics DE9600 modem card

similar to the PacComm MC-NB96.

MFJ MFJ-9600 9600 bps modem card ($110)

similar to the others.

The new 9600 bps TAPR modem kit ($80).

The new design has all of the features of the G3RUH, plus a few

enhancements. It is attractive for repeater use, since it

includes provision on the board for bit regeneration/FIFO

buffering ($15 extra for the parts). The first rev of the board

in 1992 had a few problems, and some mods were needed for best

performance. A new rev which incorporates the fixes became

available in early 1993. TAPR members get a 10% discount on

purchases.

DRSI DPK-9600 ($250).

This is a G3RUH-compatible modem and TNC-2 clone (10 MHz clock)

housed in one box.

The G3RUH and TAPR modems can plug directly into a TNC modem

disconnect header as a daughter board, or be connected externally via

a ribbon cable.

Data Interfaces for 9600 bps

For 9600 bps, the usual interface is a TNC. If you don't already have

a TNC, it's worth considering a PC bus interface card like the PI2 or

the PackeTwin. They are a better investment since they will not become

obsolete if you upgrade to higher speeds than 9600. In fact, many

people have reported results with TNCs that were much less than

theoretical maximums, even at 9600 bps. The faster the TNC clock rate

the better: 4.9MHz should be considered an absolute minimum. If you

really feel like investing in a TNC, you might consider those which

come with a built-in 9600 bps modem: the AEA PK-96 ($200 range), the

PacComm TNC-NB96 (listed in their Spring 1994 catalog at $295, but

that seems badly overpriced), or the DRSI DPK-9600 mentioned above.

Ottawa PI2 card ($125 plus shipping).

It provides a DMA port which handles 56 kbps with ease, even

with a 4.77 MHz XT-class machine. All you need to add is the

cable to the modem. The main limitation of the card is that it

does not support full-duplex operation, but full-duplex

operation is rare (especially amongst end users). The PI2 also

has a low-speed port and can be populated with an on-board 1200

bps modem (kit available for $30). The board can be used with

any DOS version of KA9Q NOS, or with Linux.

For more information about the PI2 Card, send an e-mail to

pi-info@hydra.carleton.ca for an automated blurb.

Gracilis PackeTwin-PC card ($225).

Like the PI2, it provides a DMA port for the 56 kbps modem and

an interrupt-driven port for lower-speed modems. The DMA port

supports full-duplex operation. The Kantronics 9600 bps modem

can be piggybacked on the card.

DRSI PCPA Type 1296 ($290).

An interrupt-driven PC-plug-in card with onboard 9600 bps and

1200 bps modems.

Radios for 9600 bps

A standard NBFM radio is typically used. To interface to the modem,

the radio must have a direct FSK modulator, discriminator output, an

IF with sufficient bandwidth and reasonable phase characteristics, and

fast t/r switching. Some radios are usable with just a few

modifications to bring out the required signals, others may need more

extensive mods such as adding a varactor FM modulator, and still

others are almost completely unusable due to their IF characteristics

or slow t/r switching. There are a few radios designed specifically

for digital service which require no mods:

2M:

Alinco DR-1200TH (approx. $300).

This is a version of the DR-1200T 25W synthesized radio which

has been modified for 9600 bps operation.

DRSI offers a "matched set" ($550)

consisting of their DPK-9600 TNC/modem and a modified (by DRSI)

Alinco DR-1200T.

MFJ MFJ-8621 VHF Data Radio ($120, crystals other than 145.01 extra),

5W crystal-controlled transceiver for 9600 bps or less.

70CM:

Tekk KS900L Data Radio ($120, plus $25 for custom crystals),

2W output, one channel, crystal controlled (430-450 MHz).

Available directly from Tekk, and in packages from Gracilis and

other sources. Tekk has a more expensive model called the KS960

(in a larger, chrome case) which apparently does not work as

well at 9600 bps as the KS900L, and thus should be avoided.

Gracilis 9600 bps package ($520).

It consists of the PackeTwin card, 9600 bps modem, and Tekk

KS-900.

PacComm IPR-NB96 ($495)

This package includes a Tekk radio, modem, and Tiny-2 MK-2 TNC

housed in one box.

Both Gracilis and PacComm also sell Tekk radios separately.

Kantronics D4-10 ($359),

10W output, two channel, crystal controlled (430-450 MHz). Can

go to at least 19.2 kbps.

A large number of amateur VHF and UHF transceivers have been

successfully used for 9600 bps work. Many commercial FM radios are

also suitable; ironically, the IF filters in these radios are

typically 'better' (narrower bandwidth, steeper skirts) than in

amateur-grade equipment, which leads to inferior performance at 9600

bps (on the other hand, they also tend to have superior intermod

immunity compared to amateur rigs). The IF stages of most receivers

can be broadbanded successfully, but the degree of difficulty and

expense involved varies considerably.

A good source of information on radio interfacing and other topics

related to 9600 bps operation is the"9600 Baud Packet Handbook" by

Mike Curtis, WD6EHR. Hard copies are distributed with the TAPR

modem, and it can also be found in electronic form on some BBS's

and on the Internet. The latest version seems to 2.0 (June 94).

Summary: 9600 bps

The cost of getting something working at 9600 bps is highly variable.

If you already had a TNC and a suitable radio plus antenna, it could

be as little as $80 or so (TAPR modem). On the other hand, you can get

a "plug 'n play" package from Gracilis, consisting of a PackeTwin

interface card, DE9600 modem (piggybacks on the PackeTwin), and Tekk

radio, for about $500 - just add an antenna. You should seriously

question spending this kind of money to get 9600 bps, when you could

put together a 56 kbps setup for not much more money (but, admittedly,

considerably more effort!).

EQUIPMENT FOR 19.2 KBPS

Until recently, operation at 19.2 kbps had not received much

attention. A major reason for this is that binary FSK at 19.2 kbps

cannot be accommodated by the IF stages of NBFM receivers, nor is it

compatible with the 20 or 25 kHz channel spacing used for FM in the

amateur VHF/UHF bands. On the other hand, it makes relatively poor use

of the 100 kHz channels typically allocated for 'wideband' digital

modes. However, interest in 19.2 kbps operation has been spurred by

the appearance of the Kantronics D4-10 radio. Since it contains a

varactor modulator, plus a data slicer following the discriminator, it

can be operated in 'raw FSK' mode at 19.2 kbps without additional

modem hardware. All that is needed in addition to the radio is the

computer interface. A 'souped-up' TNC might work fairly well, but one

of the PC DMA interface boards (or maybe a DataEngine) would be

better. Running 'modemless' FSK entails some loss of performance, most

notably from the lack of data scrambling, which results in more jitter

in the recovered clock signal and thus higher bit error rates.

Kantronics also offers a 19.2 kbps modem, similar to the DE9600. The

performance difference between the 'barebones' D4-10 radios and that

which you could realize with the more sophisticated modem has not, to

my knowledge, been quantified. The GRAPES modem (see below) could also

be run at 19.2 kbps, but it would not be compatible with the

Kantronics equipment (and why would you want to throttle back a modem

that can do 56 kbps and more, to only 19.2?).

Some experiences with using the D4-10's at 19.2 kbps, using Ottawa PI

cards and DataEngines as interfaces, appear in an article by John

Ackermann, AG9V in the 11th ARRL Computer Networking Conference

Proceedings.

EQUIPMENT FOR 56 KBPS

56 kbps Modem

GRAPES (WA4DSY) modem, $250 in kit form. You also need to provide a

box for it, plus a few interconnecting cables and connectors. It

requires +/-5V power (about 0.5A @ +5V, 0.1A @ -5V). This is an RF

modem with input and output (about 1 mW) in the 28-30 MHz band,

designed for use in the bands above 220 MHz (occupied bandwidth is

about 70 kHz at 56 kbps), using standard receive and transmit

converters. The receive and transmit portions of the modem are

separately crystal-controlled , and it can run full-duplex. It is not

limited to 56 kbps - with suitable modifications, it can be made to

work at 128 kbps or more.

Data Interface for 56 kbps

Ottawa PI2 card ($125)

Gracilis PackeTwin card ($225)

Both of these cards (see descriptions above) will handle 56 kbps with

ease.

Kantronics DataEngine (~$350?).

This is essentially a higher-speed TNC with two HDLC ports that

can reportedly run at 56 kbps, and an RS-232 port that can run

at up to 19.2 kbps. The standard firmware is KA-Node or G8BPQ,

but there is now also a port of JNOS (JNOS40) by WG7J

available. The DE appears to be more useful as a small

standalone packet switch than as an interface for end users.

Gracilis PackeTen (~$1000).

This is a full-blown packet switch that runs a custom version

of KA9Q NOS. It is available in both standalone and PC bus

versions. This is the Rolls Royce of packet switch hardware.

The name is a bit of a misnomer, since one PackeTen provides

five ports - you must add another one to get ten ports. All of

the ports will handle synchronous or asynchronous rates of 1200

to 19200 bps, and three of them will do synchronous rates up to

T1 or more.

RF Equipment for 56 kbps

The RF equipment required depends on whether the links are half- or

full-duplex. There are three basic configurations in use:

(1) Half-duplex point-to-point links

An example is the Georgia backbone network. The usual RF

equipment is a Microwave Modules (220, 430 MHz) or Sinclabs

(220 MHz) transverter.

(2) Full-duplex point-to-point links

Full duplex operation is significantly more complicated, but it

is also highly desirable if you want to maximize the throughput

of a backbone link. The GRAPES modem is inherently full-duplex,

so it is only necessary to provide separate RF up- and

down-converters. The two channels may be in-band or cross-band,

using either separate antennas or duplexers. The only

full-duplex point-to-point link I'm aware of is in Chicago - it

uses PackeTen switches and operates in-band in the 70 cm band.

(3) Multiple-access networks with full-duplex repeater

In this case, an in-band or cross-band 56 kbps repeater

provides hidden transmitter-free access to a channel (or

rather, a pair of channels) by multiple 56 kbps stations. This

might just be a LAN for the power users, but it also is an

attractive means of linking a number of network nodes together,

with less complexity than multiple point-to-point links (see

the 10th ARRL Computer Networking Conference proceedings for

more details). As in the preceding case, separate receive and

transmit converters are used, usually with separate antennas

(in principle, a transverter with "split" frequency operation

could be used, but such things are hard to come by). The

stations in this network do not require full-duplex computer

interfaces, but since the RF portions have full-duplex

capability, it allows smaller txdelays to be used than in the

half-duplex case. It also allows users to observe the quality

of their signals coming back from the repeater.

The first 56 kbps full-duplex repeater went on the air in Ottawa in

January 1990. The repeater is cross-band (220.55 MHz in, 433.55 MHz

out), so users must up-convert the modem's 28-30 MHz IF output to 220

MHz, and down-convert 432 MHz to the 28-30 MHz IF input.

220 MHZ (222 MHZ IN THE US!)

Transverters and up-converters:

Sinclabs ST220-28 transverter ($329 CDN),

15W output. Sinclabs has recently withdrawn from this business,

but transverters may still be available from Bob Morton, VE3BFM

(Maple Leaf Communications).

Microwave Modules MMT220/28S transverter,

10W output. Not readily available new, but watch for used ones

on the market. You may also come across some MM clones produced

by Hans Peters VE3CRU ("Transverters Unlimited") - they are

good quality, and produce higher output (~15W).

Down East Microwave DEM222 no-tune transverter, 20W output,

1-100 mw input:

DEM222-28CK complete kit (PC board, board-mounted parts,

enclosure, connectors, heat sink, hardware, $295)

DEM222-28K board-level kit (PC board, board-mounted parts, $225)

DEM222B assembled and tested unit ($395)

SSB Electronic TV 28-220/01 transverter ($380),

100 mW output. These units have no T/R switching, so that would

have to be added externally for single-channel half-duplex

operation. On the other hand, there are separate local

oscillators provided for the receive and transmit converters,

so this looks like a good choice for in-band full-duplex or

half-duplex split operation.

Hamtronics XV4 transmit converter (kit, $79),

0.5 - 1W output. The cheapest alternative, and the power level

is adequate if you aren't too far from the repeater and have a

reasonable transmitting antenna. But you do need to find

someone with a spectrum analyzer to get it tuned up properly,

and some people have had problems taming this unit.

Down-converters:

Microwave Modules MMc220 (current price/availability unknown),

2.8 dB NF.

Advanced Receiver Research R220VDA (price/availability unknown).

This unit is in the $100 range and of high quality, but it

really needs a front-end preamp. We use one of these converters

on the Ottawa 56kb repeater, along with an ARR preamp.

Antennas:

You might get by with omni antennas, but multipath can cause poor

performance even when signal levels are high. Small yagis provide more

margin and help discriminate against multipath. A typical example is

the Cushcraft A220-7 7-element yagi (about $50).

430 MHZ

Transverters and up-converters:

Down East Microwave DEM432 no-tune transverter,

50-100mW output. This is a 3-board set, available in several

forms, and there is an optional power amplifier that provides

15W output. The local oscillator board normally has a single

oscillator for standard half-duplex operation, but a second

oscillator can be added on the board for half-duplex split or

full-duplex operation. Some options and prices:

DEM432B assembled and tested unit, including case, $275

DEM432BD as above, but set up for dual frequencies, $300

DEM432K basic kit (no case or connectors), $155

Second LO kit, $8

432PA 15W PA, assembled and tested, $180

432PACK 15W PA complete kit, $135

432PAK 15W PA basic kit (no case, connectors or heat sink), $75

Enclosure to house both DEM432K and 432PA, $25

DEM432-15S complete 15W dual-frequency transverter, $395

Microwave Modules MMT432/28S transverter,

10W output. Not readily available new, but quite a few used

ones on the market.

SSB Electronic TV 28-432 transverter ($310),

100mW output. These units have no T/R switching, so that would

have to be added externally for single-channel half-duplex

operation. On the other hand, there are separate local

oscillators provided for the receive and transmit converters,

so this looks like a good choice for in-band full-duplex or

half-duplex split operation.

Hamtronics XV4 transmit converter (kit, $79),

0.5 - 1W output. The 432 MHz version of the unit described

above.

Down-converters:

Hamtronics ($49/$69/$99 for basic kit/kit with box/wired & tested).

Quality of this unit is uncertain.

Microwave Modules MMc435.2 ($115).

Current availability unknown.

SSB Electronic K7001-10 ($180).

High quality, with a price to match.

There are other sources for units in the $100-$150 range, such as

Lunar.

1.2 GHZ

Equipment for operation of the GRAPES modem at 1.2 GHz and the other

bands above 450 MHz is a problem, due to the scarcity of converters

which have input/output at 28 MHz, not to mention reasonable power

output.

Transverters and up-converters:

Down East Microwave SHF-1240(K) "No-tune" transverter board ($149 kit,

$189 assembled):

144 MHz IF (10 mW drive required), 10 mW output. Also required

is the separate SHF-LO local oscillator board ($50 kit, less

crystal; $85 assembled). A complete transverter (transverter

board, LO board, IF PIN diode switch, packaged in a metal box)

is available for $265. Note that no RF switching is included,

so if you wanted to run half-duplex, a suitable RF T/R switch

or a circulator would be needed. NF of the down-converter is in

the 4-5 dB range. Due to the 144 MHz IF, a separate 28 MHz to

144 MHz conversion stage would be needed.

SSB Electronic USM-3 transmit converter ($210).

1W out (20 mW in). Requires external LO source (10 mW).

Although normally used with 144 MHz IF, it reportedly can be

tuned for 28-30 MHz IF input. Housed in a metal box with BNC

connectors.

Down-converters:

SSB Electronic UEK-3 receive converter ($200).

2.2 dB NF, 20 dB conversion gain. The nominal LO frequency is

1152 MHz, for conversion of the 1296-1298 range to 144-146 MHz.

An LO output port is provided for driving the USM-3 transmit

converter. Housed in a metal box with BNC connectors.

Power amplifiers:

Pauldon (kit, $165):

18W out for 1W in.

Down East Microwave 2318PAM ($205):

18W out for 1W in. Also available in kit form.

SSB Electronic PA 2310 ($250):

10W out for 0.5W in (a 20W out version is $300).

Antennas:

Although loop yagis are commonly used at 1.2 GHz ($89 kit, $109

assembled for the 45-element loop yagi from Down East Microwave), a

better choice for linking would probably be the Tonna 23-element yagi

(about $70).

OTHER CONSIDERATIONS (APPLIES TO ALL BANDS):

The receive converters have very broad front ends, and some additional

bandpass filtering will often be needed. A single cavity (or helical

resonator front-end filter, in the case of separate receive

converters) should do the trick in most cases. There is also a design

available for a home-brew 28-30 MHz bandpass filter for the modem

front end. This might eliminate the need for a front-end filter, but

it depends on your receiving environment.

56 kbps Summary

The cost of a 56 kbps station is a bit hard to pin down, given all the

variables. As an example, we'll consider a station for the Ottawa 56kb

LAN. The modem kit and a PI2 card will set you back about $375. The

rest depends on the choice of rf stuff. The total will vary from about

$500 to $800. The "low road" is using the Hamtronics kits and

scrounging up things such as boxes for them and the modem, home

brewing the antennas, etc. The "high road" is buying higher-quality

assembled and tested gear, such as the Sinclabs transverter and the MM

receive converter. If you can find some good used gear, the total

should be closer to $650. Getting on 56 kbps is certainly a more

challenging project than plug 'n play 9600, but the rewards are

greater too.

THE PACKET FRONTIER: EQUIPMENT FOR SPEEDS GREATER THAN 56 KBPS

The following is a very rough first cut at summarizing the current

situation for packet at speeds greater than 56 kbps. Commercial

availability of radio and modem equipment for these speeds is nil, so

this is really the domain of the true experimenter!

Data Interface

There is, at least, some hardware that can sink/source data at these

rates:

Ottawa PI2 card: up to 500 kbps, maybe more

Gracilis PackeTwin: up to 1 Mbps claimed

Gracilis PackeTen: up to 4 Mbps (aggregate)

RF Hardware for >56 kbps

GRAPES Modem

The GRAPES Modem can run at higher speeds than 56 kbps. In his 1987

CNC article, Dale WA4DSY mentions rates up to 120 kbps. I've run a

pair of modems back-to-back at 125 kbps. Most of the changes are

straightforward (e.g., changing the baud rate generator crystal from

3.579 MHz to 8 MHz), but there are two problem areas. Standard 555

timer chips won't work reliably above 100 kHz, so the receiver clock

recovery PLL circuit needs more than just a resistor or capacitor

change. Replacement of the 555 with a TI TLC555 does the trick. The

other problem is the 455 kHz IF filter: it can be retuned so that the

modem works at 125 kbps, but the eye pattern isn't very satisfactory.

A new filter design will be needed for decent on-the-air results.

N6GN 250-500 Kbps RF Modem

Glenn Elmore N6GN (glenne@sr.hp.com) has designed an RF modem for the

902 MHz band which is intended for use at rates in the 250 to 500 kbps

range. The modem is crystal-controlled, has about 12W output, and

extremely fast turnaround time. Note that this is an FSK modem, and is

not at all similar to the spread spectrum "wireless ethernet" systems

which are available for this band. Glenn is currently running some of

these units at 230.4 kbps, using Ottawa PI2 cards. No PC boards or

construction plans are available at the moment. Units for 1.2 GHz

have been mentioned, but I'm not sure if they exist. More info on

this work and the 10 GHz equipment described in the next section can

be found on the Internet at col.hp.com:~/hamradio/packet/n6gn directory.

High-Speed Packet at 10 GHz

Glenn Elmore and friends (Kevin Rowett N6RCE, Bdale Garbee N3EUA, etc)

have also designed an X-band (10 GHz) full-duplex packet data link

using inexpensive gunnplexor modules that runs at a nominal 2 Mbps

rate. Details can be found in recent editions of the ARRL Handbook. I

haven't heard of any such systems actually being put into service yet,

but there is a source for the PC boards:

Date: Tue, 10 Aug 93 16:30:44 CDT

From: bob@lachman.com (Bob Van Valzah)

To: glenne@srlr12.sr.hp.com, bdale@col.hp.com, ron@chaos.eng.wayne.edu,

tcp-group@ucsd.edu

Subject: 2 MBPS 10 GHz Link Boards Available

Receiver boards are now (at last!) available for the n6gn designed,

2 MBPS, 10 GHz microwave link as described in the ARRL Handbook and

other places.  You'll still need to make the 1st IF board yourself,

but trust me, that's easy.  I didn't believe Glenn at first, just

follow the procedure in the article and it works!

The receiver board is 4-3/4 x 5-3/4", double sided, and drilled,

*but not plated through* (hence you'll have to solder top and bottom

leads in some cases).  This shouldn't be a big problem because there

are no signal traces on the component side--it's all ground plane.

Folks who helped make these boards available (though they may not

all want to admit it :-) are Glenn Elmore n6gn (of course), John

Conner wd0fhg, Ron Atkinson n8fow, Fred Reimers kf9gx, Bdale Garbee

n3eua, and Jon Bloom ke3z.

They're available from:

Fred A. Reimers kf9gx

FAR Circuits

18N640 Field Ct.

Dundee, IL 60118-9269

for $15.  Fred just shipped 8 to me for $5 postage, but I'm not sure

what the rate would be for smaller quantities.

T1 Modem

Clint Turner KA7OEI (ka7oei@uugate.wa7slg.ampr.org) has designed a T1

rate (1.536 Mbps) modem, intended for use in the packet backbone

network in Utah. This modem is designed for full-duplex operation and

produces and demodulates a baseband-type signal. It was designed for

FM transmitter/receiver systems with a baseband bandwidth of at least

1.5 MHz.

SOURCES

Advanced Receiver Research

Box 1242

Burlington, CT 06013

203-582-9409

Down East Microwave

RR 1, Box 2310

Troy, ME 04987

207-948-3741 Fax: 207-948-5157

DRSI (Digital Radio Systems Inc)

2065 Range Road

Clearwater, FL 34625

813-461-0204 Fax: 813-447-4369

Note: DRSI announced in late 1994 that they are no longer

producing and selling products for the amateur radio market.

Gracilis Inc

623 Palace Street

Aurora, IL

708-801-8800 Fax: 708-844-0183

Email: info@gracilis.com

GRAPES Inc.

P.O. Box 636

Griffin, GA 30224

Email: ka4byp@mail.radio.org

Hamtronics Inc.

65-Q Moul Road

Hilton, NY 14468

716-392-9430 Fax: 716-392-9420

Kantronics

1202 E. 23rd Street

Lawrence, KS 66046

913-842-7745 Fax: 913-842-2021 BBS: 913-842-4678

Maple Leaf Communications (Bob Morton, VE3BFM)

R.R. 1

Everett, ON, Canada L0M 1J0

705-435-0689

MFJ Enterprises Inc

PO Box 494

Mississippi State, MS 39762

1-800-647-1800 (order) 1-800-647-8324 (tech info) Fax: 601-323-6551

Ottawa Amateur Radio Club

Packet Working Group

P.O. Box 8873

Ottawa, ON, Canada K1G 3J2

Email: bm@hydra.carleton.ca (for PI2 info: pi-info@hydra.carleton.ca)

PacComm Packet Radio Systems Inc

4413 N. Hesperides Street

Tampa, FL 33614-7618

813-874-2980 Fax: 813-872-8696

Pauldon Associates

210 Utica Street

Tonawanda, NY 14150

716-692-5451

SSB Electronic USA

124 Cherrywood Drive

Mountaintop, PA 18707

717-868-5643

Tekk, Inc

226 N.W. Parkway

Kansas City, MO 64150

1-800-521-8355 (orders) 816-746-1098 Fax: 816-746-1093

Tucson Amateur Packet Radio

P.O. Box 12925

Tucson, AZ 85732-2925

602-749-9479 (1700-2200UTC, Tue. - Fri.) Fax: 602-749-5636

Email: tapr@tapr.org

[eof]