SPREAD SPECTRUM RADIOS

Cylink, Proxim, FCC Part 15, etc.

Compiled by WÝTN

Rev. 1, 1-3-95

Cylink

In article <9411092031.AA04204@cacfs.ks.erg.sri.com>

rush@sparkyfs.erg.sri.com (David Rush) writes:

> Cylink sells a few RF modems for 902-928 MHz that fall under the spread-

> spectrum no-license rules.  They've run units as much as 30 miles under

> ideal conditions (between mountain-top sites, big antennas, LOS).

> They have 64 kbps, 128 kbps, and 256 kbps units.  I've used the 128 kbps

> units for mobile VTC.

> You simply plug in your antenna (units come with rubber ducks!), the

> power supply, and your data (EIA-530/RS-449 or V.35, as I recall),

> and go.

>

> Try them at 708-377-7300 (I think that's the Chicago area office).

>

> David

--------------------------------------------------------------------------------------

FREEWAVE

Date: 11 Dec 1994 05:59:10 GMT

From: Jonathan Sawyer <76360.3417@CompuServe.COM>

Subject: Spread Spectrum

Check out Freewave Technologies SS Modem.  They are a lot less

expensive than Cylink modems and maintain a 115.2 KB throughput.

In fact I am writing this on my office computer from my home

computer 22 miles away using a FreeWave modem.  FreeWave is in

Boulder Co. Their number is 303-444-3862

------------------------------------------------------------------------

[1-3-95]

Date: 2 Jan 1995 15:38:31 GMT

From: k9cw@prairienet.org (Andrew B. White)

Subject: Ricochet Spread Spectrum modems...

In a previous article, pipenet@bonk.io.org (pipeNet) says:

>

>Does anyone have any experience using these? Any comments on the claimed

>operating distance, or advice on replacement aerials to make them perform

>better? What about data throughput?

>

I do not have experience with the Ricochet units, but I am currently

evaluating a pair of FreeWave DGR-115's.  These units will run up to 115k

on the serial async channel, and I am using two units over a 15 mile path

with 5 element yagis at about 20 feet.  The manufacturer claims that the

units will go 20 miles line of site between top mounted rubber duck antennas!

All powered by a 12VDC/.5 amp supply.  FreeWave is a relatively new company

in Boulder, CO.

73, Drew

*-----------------------------*-------------------------------------*

|    Andrew B. White  K9CW    |    internet: k9cw@prairienet.org    |

|    ABW Associates, Ltd.     |   phone/fax: 217-643-7327           |

*-----------------------------*-------------------------------------*

PROXIM

> On page 19 of November's "Circuit Cellar INK", there is an add for

> Timeline selling Proxim spread-spectrum RF modems for $99. It says:

>

>         The Proxlink is a small communication device that replaces the

>         use of an RS-232 cable hook-up with wireless (Radio Frequency)

>         technology. Each module combines the functions of an RS-232

>         compatible modem (or any other RS-232 device) and a

>         transmitter/ receiver operating in the 902-928 MHz band. Indoor

>         range of 500 feet possible using standard 1/4 wave antenna.

>         Units support data rate to 19.2 KBaud (full duplex) and use

>         multiple channels to allow for non-interfering overlapping

>         systems. Message format is 8 data bits, no parity, and 1 stop

>         bit. Each unit requires a 5VDC supply at a maximum of 230 mA. A

>         wide variety of configurations can be accomodated with

>         parameters stored in non-volatile memory (EEPROM).

>         Configuration changes are supported by menu-driven, on-board

>         software.  Installation schematics and application details are

>         available. Original cost of these units is around $600.

>

> These are Part-15 equipment. I wonder if these units are illegal for

> use in Amateur service because they don't use one of the few spreading

> sequences that the FCC has approved for amateur use. That's the case

> with most commercial spread-spectrum equipment that I've heard of.

>

> If you want to call Timeline to pursue this, their technical information

> phone number is 310-784-5488. Of course I don't have anything to do with

> Timeline, I simply thought you might be interested in this equipment.

>

>         Thanks

>

>         Bruce Perens

We bought some of these to play with at work. Haven't had a chance to play

with them yet, but some things ought to be clarified:

- The serial link is TTL. A schematic is included showing how to build

an RS-232 converter using a MAX-232.

- No enclosure, no antenna. Funny little antenna jack.

As for amateur radio use: don't put too big of an antenna or add a power

amplifier. Then you would not be using it in the amateur radio service,

and suddenly the DS coding is legal. This radio would be real tempting to

mount on the tower. An example in the manual shows a pair of them connected

via serial ports back-to-back as a repeater. They even have a maintenance

protocol for reconfiguring the remote radio(s).

One day I hope to find time to apply power to mine.

73, David N4HHE

dkelly@nebula.tbe.com

---------------------------------------------------------------------

The Timeline Proxim SS modems are the OEM version of the ones I was carting

around and playing with at the ARRL DCC this last August. OEM in this

case means no case, antenna or apparent RS-232 conversions.

Range with a 1/4 antenna covered most of the motel. I'd guess that to be about

500 feet as advertised. Throughput was between 1200-1600 bytes a second.

This was with SLIP links operating both units in broadcast mode.

Interestingly enough, while the serial rate is limited to 19.2kbaud, the

radio data rate is between 161-262 kbaud. I called proxim to see if

they would release the source (yeah right, but hey?) to see if I could

bump the UART serial rate up, but that they didn't seem to want to. The

processor in them I seem to recall is a 6502.

I'm told by a fellow who makes SS his business that the Proxims are

poor performers compared to some other units, but for $99 it certainly

is tempting.

Timeline claims they are shipping schematics with them. Anyone confirm this?

Jeff

SMTP     - wb8wka@wb8wka.ampr.org [44.102.16.16] |  Operating

Internet - wb8wka@merit.edu , jking@merit.edu    |       on the edge

AX25     - wb8wka@wu8c.#SEMI.MI.USA.NOAM         |              of Chaos

--------------------------------------------------------------------------------------

> Timeline claims they are shipping schematics with them. Anyone confirm this?

hi hi hi hi. Not by my yardstick. The manuals are pretty good, but one

manual is for the "finished product" and the other is part of the OEM

configuration software stuff. Tells you how to remove "Proxim" from the

setup screen and replace it with your name, etc.

What passes for schematics is (4) 8.5"x11" xeroxes of the 4 quadrants of

a C sized schematic. Don't have it in front of me, but as I recall all that

was useful (well, maybe useful) was the hand drawn additions showing how

to add a Maxim 232 (or 233?) RS-232 driver.

Jeff King mentioned that he thought Proxim was using a 6502, and that is

true. Never saw a 65C02 in PLCC before. Probably never again. And it wasn't

even a special hi-integration microcontroller version of the 65C02. Couldn't

tell what UART was used, guess it got integrated into one of the special

chips.

73, David N4HHE

dkelly@nebula.tbe.com <<-- use this address

--------------------------------------------------------------------

I have a couple of the proxim rf modems working.  The schematic that is

provided with the modems uses a MAX238 chip to convert the 5 volt signals

to RS232 levels.  Unfortunately, that is not a common chip.  I used the

readily available MAX232 and only wired the TX and RX data lines.  That is

enough to get it working.  The framing character is programmable.  I set it

to 192 and KISS works like a charm.  I got ftp speeds of 500 cps with JNOS

for Linux talking to an old slow laptop.  I'm curious to see what kind of

range the units have.  They work short distances with a 4 inch piece of

wire-wrap tacked to the back side of the SMA socket.

------------------------------------------------------------------------

The antenna connector is listed as BFA in one place and SMA in another.

I plan to run a short piece of 50 ohm coax from the back of the connector

(what ever it is) to a type N mounted on the metal box I'm putting the

modem in.

----------------------------------------------------------------------

KC5AEX and I finished up the a link built with proxim modems today.

The thruput is impressive, about 1600 characters per second doing

point-to-point FTP.  The link we built is only about one-half mile

but is not line-of-sight - there is a low hill between the two nodes.

The line-of-sight range appears to be on the order of 3 to 5 miles with

reasonable antennas, in our case a base station vertical and a small

yagi.  The total cost of the link was about $500 including the modems,

100' of 9913, two new antennas, and misc. parts.  IMHO, pretty good

performance for the price.

------------------------------------------------------------------------

Licensing Considerations

Bruce Perens said... [re Proxim]

>> These are Part-15 equipment. I wonder if these units are illegal for

>> use in Amateur service because they don't use one of the few spreading

>> sequences that the FCC has approved for amateur use. That's the case

>> with most commercial spread-spectrum equipment that I've heard of.

Then David N4HHE said...

>As for amateur radio use: don't put too big of an antenna or add a power

>amplifier. Then you would not be using it in the amateur radio service,

>and suddenly the DS coding is legal. This radio would be real tempting to

>mount on the tower. An example in the manual shows a pair of them connected

>via serial ports back-to-back as a repeater. They even have a maintenance

>protocol for reconfiguring the remote radio(s).

For which I reply...

This is taken from the 7th Annual One Day Seminar on Spread Spectrum

presented by the Vehicular Technology Society of the IEEE, dated June 22,

1993.  From a talk given by Graham Barnes, Director of Marketing and Sales,

Western Multiplex.

"The regulatory basis of Part 15 spread spectrum links is provided by the

FCC Rule Part 15.247.

..

Transmitters operating under Part 15.247 may intentionally radiate only in

the ISM (Industrial, Scientific and Medical) band and may only use frequency

hopping or direct sequence techniques.  No other spreading techniques are

permitted.  There are three microwave frequency bands allocated to this service:

902 - 928 MHz (26 MHz bandwidth)

2400 - 2483.5 MHz (83.5 MHz bandwidth)

5725 - 5850 MHz (125 MHz bandwidth)

..maximum transmitter EIRP which is limited to +6dBW (1 watt into a 6dBi

gain antenna).  However, users may take advantage of transition provisions

under which the EIRP limit is modified to a 1 W RF output power into any

antenna.  Products that are certified by June 23, 1992 and manufactured up

until June 23, 1994 fall under the transition provisions and equipment users

with these products can operate at this power level indefinitely."

As for the Amateur Radio Service read up on section 97.311 for SS

techniques. In this section it describes frequency hopping and direct

sequence regulations.  Under DS there are three allowed spreading codes.

Needless to say, the part 15 devices that are being sold are not likely to

be using one of these three spreading codes.  Thus it is better to operate

these devices under the part 15 provision.

Steve, N7HPR

srbible@cs.nps.navy.mil

Naval Postgraduate School,  Monterey, CA 9394

----------------------------------------------------------------------